| Mfg. Date | : |  | Exp. Date | : |  |
| --- | --- | --- | --- | --- | --- |
| Test Request Shet No. | : |  | A.R. No. | : |  |
| Batch Size | : |  | Quantity Sampled | : |  |
| Received Date | : |  | Testing Date | : |  |
| Reference Specification/MOA No. | : | SPEC/FG00038/01 (IP) MOA/FG00038/01 (IP) | Issuance Stamp by QA |  |  |
| Completion Date | : |  |  |  |  |

| Sr. No | Tests |  | Results | Limits |
| --- | --- | --- | --- | --- |
| 1. | Description |  |  | A white crystalline powder. |
| 2. | Solubility |  |  | Freely soluble in water; very slightly soluble in ethanol (95%); practically insoluble in ether. |
| 3. | Identification |  |  |  |
|  | By IR |  |  | IR spectrum of Glycine sample should be concordant with IR spectrum of Glycine working standard. |
|  | By chemical test |  |  | The resulting solution is violet with a greenish yellow fluorescence. After a few minutes the solution becomes orange and then yellow and the intense fluorescence remains. |
| 4. | Appearance of solution |  |  | A 10.0% w/v solution in carbon dioxide-free water is clear & not more intensely coloured than reference solution YS7. |
| 5. | pH |  |  | 5.9 to 6.3 |
| 6. | Heavy metals |  |  | NMT 10 ppm |
| 7. | Chlorides |  |  | NMT 100 ppm |
| 8. | Sulphated ash |  |  | NMT 0.1% |
| 9. | Loss on drying |  |  | NMT 0.5% |
| 10. | Assay |  |  | 98.5% to 101.5% |
| Additional Tests |  |  |  |  |
| 11. | Sieve analysis |  |  | As per customer requirement |
| 12. | Bulk Density |  |  | As per customer requirement |
| 13. | Tapped Density |  |  | As per customer requirement |
| 14. | Elemental impurities by AAS (As per ICH Q3D) |  |  |  |
|  | Lead (Pb) |  |  | NMT 0.5 µg/g |
|  | Arsenic (As) |  |  | NMT 0.5 µg/g |
|  | Mercury (Hg) |  |  | NMT 0.1 µg/g |
|  | Cadmium (Cd) |  |  | NMT 0.5 µg/g |
|  | Cobalt (Co) |  |  | NMT 5.0 µg/g |
|  | Vanadium (V) |  |  | NMT 10.0 µg/g |
|  | Nickle (Ni) |  |  | NMT 20.0 µg/g |
|  | Lithium (Li) |  |  | NMT 25.0 µg/g |
|  | Antimony (Sb) |  |  | NMT 9.0 µg/g |
|  | Copper (Cu) |  |  | NMT 30.0 µg/g |
| 15. | Foreign matter |  |  | Should be absent in 5 gm sample |
| 16. | Residual solvents |  |  | Methanol: NMT 3000 ppm |
| 17. | Ethylene oxide |  |  | NMT 0.1 ppm |
| Microbiological parameters |  |  |  |  |
| 17. | TAMC |  |  | NMT 103 CFU/ g |
|  | TYMC |  |  | NMT 102 CFU/ g |
|  | Pathogen in general |  |  | Should be absent |
|  | Bacterial endotoxin test |  |  | NMT 5 EU/gm |

Note: Above mentioned product complies / does not comply as per IP specification.

|  | Analyzed By | Checked By | Approved By |
| --- | --- | --- | --- |
| Sign |  |  |  |
| Date |  |  |  |

| 1.0 | Description: |  |
| --- | --- | --- |
| Procedure: Take about ___________ (2-3) g of substance into Petri dish. Check product for colour. Observation: _________________________________________________________________________ Acceptance criteria: A white to off-white crystalline powder odourless. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |
| 2.0 | Identification: |  |
| A. | By IR Spectrophotometry: |  |
| Instrument Used: Balance ID: _________________ Date of Calibration: _______________ Use Before: __________________ FTIR ID: __________________ Date of Calibration: _______________ Use Before: __________________ Blank preparation: Take 100.0 mg of KBr to be analysed. Load blank sample into DSR unit and take IR by following steps carried out for the background. Standard preparation: Prepare standard by mixing about 100.0 mg of KBr and ______(3.0) mg of Glycine working standard. Load standard into DSR unit and take IR. Sample preparation: Prepare sample by mixing about 100.0 mg of KBr and _______(3.0) mg of substance to be analysed. Load sample into DSR unit and take IR. Procedure: Ensure that instrument is clean and free from dust. Switch ON the instrument and log in with respective ID as an authorized analyst. Set the desire instrument parameters. Allow the IR to stabilize IR takes some time for stabilization. After successful initialization, Install DSR unit in the IR. Clean the sample holder. Prepare fine powder of KBr in the mortar. Fill KBr powder into sample holder. Wipe excess powder with tissue paper and fix it to DSR unit. Select the method of background. Method parameters should be loaded and change the parameters as require. Now IR is ready for taking the background and scan the background. Take out sample holder from DSR unit. Discard KBr and clean the unit and analysed blank, standard and sample Load blank sample into DSR unit and take IR by following steps carried out for the background. Select IR graph of standard and sample, Purity index or comparison spectra shall be opened as follow. Compare purity index with established limit to conclude the IR result. Observation: - _______________________________________________________________________________________ _______________________________________________________________________________________ - Correlation between sample and standard IR spectrum: _____________ Acceptance Criteria: - IR spectrum of sample should be concordant with IR spectrum of standard. - Correlation between sample and standard IR spectrum: NLT 0.95 Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |
| B. | By chemical test: |  |
| Instrument Used: Balance ID: __________________ Date of Calibration: ________Use Before: __________________ Solution / Reagents Used: ID of 2% w/v resorcinol: _________________ Date of preparation: ___________Use Before: _____________ ID of 2M sodium hydroxide: _______________Date of preparation: ___________Use Before: ______________ Procedure: Dissolve ________(50.0) mg in 5 ml of water, add 1 ml of sodium hypochlorite solution (3% Cl), boil for 2 minutes, add 1 ml of hydrochloric acid and boil for 4 to 5 minutes. To the resulting solution add 2 ml of hydrochloric acid and 1 ml of a 2% w/v solution of resorcinol, boil for 1 minute, cool, add 10 ml of water and mix. To 5 ml of this solution add 6 ml of 2 M sodium hydroxide. Observation: _____________________________________________________________________________ _________________________________________________________________________________________ Acceptance Criteria: The resulting solution is violet with a greenish yellow fluorescence. After a few minutes the solution becomes orange and then yellow and the intense fluorescence remains. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |
| 3.0 | Appearance of solution: |  |
| A. | As per IP: |  |
| Instrument Used: Balance ID: __________________ Date of Calibration: _______________ Use Before: __________________ Procedure: Compare Solution A with water. Note: If Solution A is not as clear as water, then proceed for further opalescence method. Reagents / Solutions Used: ID of 10% Hexamine: _______________ Date of Preparation: _____________Use Before: ________________ Special Reagents: Standard Suspension: Solution ID: _______________ Date of Preparation: _____________Use Before: ________________ Prepare the diluted standard suspension by diluting 15 ml of the well mixed standard suspension to 1000 ml with water. The standard suspension should be used within 24 hours of preparation. Opalescence Standards: Prepare opalescence standards OS1 by mixing aliquots of standard suspension with water as indicated in Table 1. Each opalescence standard should be shaken well before use. Opalescence Standard Table 1: Opalescence Standard; Standard Suspension (ml); Water (ml). OS1; 5.0; 95.0. Procedure: Transfer to a flat-bottom test tube of neutral glass, 15 to 25 mm in diameter, a suitable volume of the solution under examination such that the test-tubes is filled to a depth of 4-tubes is filled to a depth of 40 mm. Into another matched test-tube add the same volume of the freshly prepared opalescence standard. After 5 minutes compare the contents of the test tubes against a black background by viewing under diffused light down the vertical axis of the tubes. Clarity of Opalescence: Express the degree of opalescence in terms of the opalescence standard. A liquid is considered as clear if its clarity is the same as that of water or of the solvent used for preparing the solution under examination or if its opalescence is not more than that of opalescence standard OS1. Observation: ________________________________________________________________________ Acceptance criteria: Solution A is clear. B. Colour of Solution Procedure: Compare the Solution A with water. If it has same appearance as water or solvent used for preparation of solution, it is considered as colourless otherwise proceed for reference solution BS8. Special Reagents: Ferric Chloride Colorimetric Solution (FCS) Solution ID: _______________ Date of Preparation: _____________Use Before: ________________ Cobaltous Chloride Colorimetric Solution (CCS) Solution ID: _______________ Date of Preparation: _____________Use Before: ________________ Cupric Sulphate Colorimetric Solution (CSS) Solution ID: _______________ Date of Preparation: _____________Use Before: ________________ Reference Solution: Prepare by mixing the volumes of colorimetric solutions and hydrochloric acid (1 % w/v HCL) as indicated in following table. Colour of reference solution; Reference solution; FCS (ml); CCS (ml); CSS (ml); Hydrochloric Acid (1%w/v) (ml). Yelloow; YS7; 0.8; 0.2; 0; 99.0. Procedure: Transfer to a flat-bottom test tube of neutral glass, 15 to 25 mm in diameter, a suitable volume of a liquid been examined such that the test-tube is filled to a depth of 40 mm. Into another matched test-tube add the same volume of water or of the solvent used for preparing the solution being examined or of the reference solution. compare the colours in diffused light viewing vertically against a white background. Colourless Solution: A solution is considered colourless if it has the same appearance as water or the solvent used for preparing the solution or is not more intensely coloured than reference solution YS7. Observation: ________________________________________________________________________ Acceptance criteria: Solution A is colourless. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |
| B. | As per BP: |  |
| Instrument Used: Balance ID: __________________ Date of Calibration: _______________ Use Before: _________________ Solution S: Dissolve _________(5.0) g in carbon dioxide free water and dilute to 50 mL with the same solvent. Clarity of Solution: Compare clarity of Solution S with water. If Solution S has clarity of water it is clear otherwise compare opalescence of Solution S with reference Suspension-I prepared as below. Hydrazine sulfate solution: Solution ID: __________________ Date of Preparation: ____________ Use Before: _______________ Primary opalescent suspension (formazin suspension): Solution ID: __________________ Date of Preparation: ____________ Use Before: _______________ Standard of opalescence: Dilute 15.0 mL of the primary opalescent suspension to 1000.0 mL with water R. This suspension is freshly prepared and may be stored for up to 24 h. Reference Suspension-I: Mix 5.0 ml of water and 95.0 ml of standard opalescence in Nessler cylinder. Reference Suspension-II: Mix 10.0 ml of water and 90.0 ml of standard opalescence in Nessler cylinder. Procedure: Using identical test-tubes of colourless, transparent, neutral glass with a flat base and an internal diameter of 15-25 mm, compare the liquid to be examined with water or reference suspension freshly prepared as described below. Ensure that the depths of the layers in the 2 test-tubes are the same (about 40 mm). Compare the liquids in diffused daylight 5 min after preparation of the reference suspension, viewing vertically against a black background. System suitability: The diffusion of light must be such that reference suspension I can readily be distinguished from water R and that reference suspension II can readily be distinguished from reference suspension I. Colour of Solution: Compare colour of Solution S with water. If Solution S has appearance of water it is colourless otherwise compare colour of Solution S with reference solution Y7 prepared as below. Reagents: Yellow solution: Solution ID: __________________ Date of Preparation: ____________ Use Before: _______________ Red solution: Solution ID: __________________ Date of Preparation: ____________ Use Before: _______________ Blue primary solution: Solution ID: __________________ Date of Preparation: ____________ Use Before: _______________ Standard solutions: Using the 3 primary solutions, prepare the standard solution as follows: Volume in millilitres. Standard solution; Yellow solution; Red solution; Blue solution; HCl (10 g/L HCl). Y (Yellow); 2.4; 0.6; 0.0; 7.0. Volumes in milliliters. Reference Solution; Standard solution Y; Hydrochloric acid (10 g/L HCl). Y7; 2.5; 97.5. Procedure: Using identical tubes of colourless, transparent, neutral glass with a flat base and an internal diameter of 15 mm to 25 mm, compare the liquid to be examined with water R or the solvent or the reference solution Y7, the depth of the layer being 40 mm. Compare the colours in diffused daylight, viewing vertically against a white background. Observation: __________________________________________________________________________. Acceptance Criteria: Solution S is clear and not more intensely coloured than reference solution Y7. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |
| 5.0 | pH (IP/BP): |  |
| Instrument Used: Balance ID: __________________ Date of Calibration: ________Use Before: __________________ Procedure: Dissolve ________ (5.0 g) of sample to 100 mL with carbon dioxide-free water. Measure pH of this solution. Observation: ___________________ (As per IP) ___________________ (As per BP) Acceptance criteria: 5.9 to 6.3 (As per IP) 5.9 to 6.4 (As per BP) Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |
| 6.0 | Heavy metals (IP): |  |
| Instrument Used: Balance ID: __________________ Date of Calibration: _______________ Use Before: _________________ Reagents / Solutions Used: ID of std. solution (20 ppm Pb):_____________Date of Preparation:___________Use Before: ____________ ID of dilute acetic acid:_____________Date of Preparation:___________Use Before: ____________ ID of dilute ammonia solution:_____________Date of Preparation:___________Use Before: ____________ ID of buffer solution:_____________Date of Preparation:___________Use Before: ____________ ID of Thioacetamide solution:_____________Date of Preparation:___________Use Before: ____________ ID of 1M Sodium hydroxide:_____________Date of Preparation:___________Use Before: ____________ ID of Glycerin (85%) solution:_____________Date of Preparation:___________Use Before: ____________ Standard solution: Into a 50 ml Nessler cylinder pipette 1.0 ml of lead standard solution (20 ppm Pb) and dilute with water to 25 ml. Adjust with dilute acetic acid or dilute ammonia solution to a pH between 3.0 and 4.0, dilute with water to about 35 ml and mix. Thioacetamide Reagent: Add 1 ml of mixture of 15 ml of 1M sodium hydroxide, 5 ml of water and 20 ml of Glycerin (85%) to 0.2 ml of thioacetamide solution, heat in a water bath for 20 seconds, cool and use immediately. Test solution: Into a 50 ml Nessler cylinder place 25 ml of the solution prepared for the test dissolve the _________(2.0) g of the substance under examination in sufficient water to produce 25 ml. Adjust with dilute acetic acid or dilute ammonia solution to a pH between 3.0 to 4.0, dilute with water to about 35 ml and mix. Procedure: To each of the cylinders containing the standard solution and test solution respectively, add 2 ml of buffer solution and 1.2 ml of thioacetamide reagent, dilute to 50 ml with water and mix. Allow to stand for 5 minutes and view downwards over a white surface, the colour produced with the test solution is not more intense than that produced with the standard solution. Observation: ________________________________________________________________________ Acceptance criteria: 10 ppm Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |
| 7.0 | Chlorides (IP/BP/USP): |  |
| A. | As per BP: |  |
| Instrument Used: Balance ID: __________________ Date of Calibration: ________Use Before: __________________ Reagents / Solutions Used: ID of std. solution (5 ppm cl):________________Date of Preparation:___________Use Before: _____________ ID of silver nitrate solution:_______________Date of Preparation:___________Use Before: _______________ Sample solution: Dissolve _________(0.67) g in water and dilute to 15 mL with the same solvent. Procedure: To sample solution add 1 mL of dilute nitric acid and pour the mixture as a single addition into a test tube containing 1 mL of silver nitrate solution R2. Prepare a standard in a same manner using 10 mL of chloride standard solution (5 ppm Cl) and 5 mL of water. Examine the tubes laterally against a black background. After standing for 5 min protected from light, any opalescence in the test solution is not more intense than that in the standard. Observation: ________________________________________________________________________ Acceptance criteria: Max 75 ppm Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |
| B. | As per USP: |  |
| Instrument Used: Balance ID: __________________ Date of Calibration: ________Use Before: __________________ Reagents / Solutions Used: ID of 0.02N HCl:________________Date of Preparation:___________Use Before: _____________ ID of silver nitrate solution:_______________Date of Preparation:___________Use Before: _______________ Procedure: Dissolve the __________(1.0) g of Glycine in 30 to 40 mL of water and, if necessary, neutralize the solution with nitric acid to litmus. Add 1 mL each of nitric acid and of silver nitrate TS and sufficient water to make 50 ml. Mix, and allow to stand for 5 minutes protected from direct sunlight. Compare the turbidity, if any, with that produced in a solution containing the 0.10 mL of 0.020 N hydrochloric acid. Observation: ________________________________________________________________________ Acceptance criteria: NMT 0.007 % Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |
| C. | As per IP: |  |
| Instrument Used: Balance ID: __________________ Date of Calibration: ________Use Before: __________________ Reagents / Solutions Used: ID of 25 ppm Cl solution:________________Date of Preparation:_____________Use Before: _______________ ID of 0.1M silver nitrate solution:_____________Date of Preparation:___________Use Before: ______________ Procedure: Dissolve the ___________(2.5) g of the substance under examination in water, or prepare a solution as directed in the individual monograph and transfer to a Nessler cylinder. Add 10 ml of dilute nitric acid, except when nitric acid is used in the preparation of the solution, dilute to 50 ml with water and add 1 ml of 0.1 M silver nitrate. Stir immediately with a glass rod and allow to stand for 5 minutes protected from light. When viewed transversely against a black background any opalescence produced is not more intense than that obtained by treating a mixture of 10.0 ml of chloride standard solution (25 ppm Cl) and 5 ml of water in the same manner. Observation: ________________________________________________________________________ Acceptance criteria: NMT 100 ppm Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |
| 8.0 | Sulfated ash (IP/BP/USP): |  |
| Instrument Used: Balance ID: __________________ Date of Calibration: ________Use Before: __________________ Muffle furnace ID: __________________ Date of Calibration: ________Use Before: __________________ Procedure: Ignite a silica or platinum crucible at 600 ± 50º for 30 minutes, allow to cool in a desiccator over silica gel or other suitable desiccant and weigh. Place the ________(1.0) g of the substance under examination in the crucible and weigh. Moisten the substance under examination with a small amount of sulfuric acid (usually 1 ml) and heat gently at as low a temperature as practicable until the sample is thoroughly charred. After cooling, moisten the residue with a small amount of sulfuric acid (usually 1 ml), heat gently until white fumes are no longer evolved and ignite at 600 ± 50º until the residue is completely incinerated. Ensure that flames are not produced at any time during the procedure. Allow the crucible to cool in a desiccator over silica gel or other suitable desiccant, weigh it again and calculate the percentage of residue. If the amount of residue so obtained exceeds the prescribed limit, repeat the moistening with sulfuric acid and ignition, as previously, for 30 minutes periods until 2 consecutive weighing’s do not differ by more than 0.5 mg or until the percentage of residue complies with the prescribed limit. The amount of substance used for the test (usually 1-2 g) is chosen so that at the prescribed limit the mass of the residue (usually about 1 mg) can be measured with sufficient accuracy. Calculation: Weight of residue (W1) x 100 Sulphated Ash= --------------------------------- Weight of sample Result: Weight of residue after first ignition (W): _____________ Weight of residue after further 30 minutes ignition (W1): _____________ Difference (W-W1):___________ (NMT 0.5 mg) % Residue on Ignition = (_________ x 100) / __________ = ___________ Acceptance Criteria: NMT 0.1 % Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |
| 9.0 | Loss on drying (IP/BP): |  |
| Instrument Used: Balance ID: __________________ Date of Calibration: _______________ Use Before: _________________ Oven ID: __________________ Date of Calibration: _______________ Use Before: _________________ Procedure: Weigh an empty weighing bottle that has been previously dried under the conditions prescribed for the substance to be examined for at least 30 min, then weigh the weighing bottle filled with the _________(1.0) g of substance to be examined. Dry to constant mass or for 2 hours. Where the drying temperature is indicated by a single value rather than a range, drying is carried out at 1050± 2 °C. Calculation: LOD = (Loss of weight x 100) / Weight of sample Result: LOD = (___________ x 100) / ___________ = ____________ Observation: _____________ (IP/BP) _____________ (USP) Acceptance criteria: NMT 0.5% (IP/BP) NMT 0.2% (USP) Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |
| 10.0 |  | Ninhydrin-positive substances (BP): |
| Test is carried out by outside laboratory using Amino acid analyzer. Report No.: ______________ Date of analysis: ________________ Observation: Each impurity: ___________ Total: ___________ Acceptance criteria: Each impurity: Max 0.10% Total: Max 1.0% Conclusion: Satisfactory / Not satisfactory Report reviewed by:____________ |  |  |
| 11.0 |  | Related Compounds (BP/USP): |
| A. |  | As per BP: |
| Instrument Used: Balance ID: __________________ Date of Calibration: _______________ Use Before: _________________ HPLC ID: __________________ Date of Calibration: _______________ Use Before: _________________ Mobile phase: Dissolve 1.4 g of sodium pentanesulfonate in 900 mL of water, adjust to pH 2.2 with phosphoric acid and dilute to 1000 ml with water. Solvent mixture: To 500 mL of the mobile phase add 1.5 mL of phosphoric acid. Test solution: Dissolve ___________(0.200) g of the substance to be examined in the solvent mixture and dilute to 20.0 mL with the solvent mixture. Reference solution (a): Dilute 1.0 mL of the test solution to 100.0 mL with the solvent mixture. Dilute 1.0 mL of this solution to 10.0 mL with the solvent mixture. Reference solution (b): Dissolve _______(80.0) mg of iminodiacetic acid (impurity A) and _______(80.0) mg of the substance to be examined in the solvent mixture and dilute to 50.0 mL with the solvent mixture. Reference solution (c): Solution ID: __________________ Date of Preparation: _______________ Use Before: _________________ Dilute 1.0 mL of reference solution c to 100.0 mL with the solvent mixture. Chromatographic system: Column: size: l = 0.25 m, Ø = 4.6 mm; Stationary phase: end-capped octadecylsilyl silica gel for chromatography (5 µm); Temperature: 25 °C Flow rate: 1.0 mL/min. Detection Spectrophotometer at 210 nm. Injection: 10 µL. Run time: 4 times the retention time of Glycine. Identification of impurities: Use the chromatogram obtained with reference solution (c) to identify the peaks due to impurities B, H and I. Relative retention with reference to Glycine (retention time = about 5.5 min): impurity A = about 0.7; impurity B = about 0.75; impurity H = about 1.7; impurity I = about 2.0. System suitability: Reference solution (b): Resolution: __________(Minimum 5.0 between the peaks due to impurity A and Glycine.) Sequence: No.; Name of Solution; No. of Injections. 1.; Reference solution b for system suitability; 1. 2.; Mobile phase; 1. 3.; Reference solution b; 1. 4.; Reference solution a; 2. 5.; Reference solution c; 2. 6.; Test solution; 2. Calculation: Unspecified impurity = (A/B x C/D x 100) Where, A= Area of unspecified impurity in test preparation B= Area of glycine in reference solution (a) C= Concentration of glycine in reference solution (a) D= Concentration of glycine in test solution Impurity (B/H/I) = (A/B x C/D x 100) A = Area of impurity (B/H/I) in test preparation B= Area of impurity (B/H/I) in reference solution (c) C= Concentration of impurity (B/H/I) in reference solution (c) D= Concentration of glycine in test solution Total impurity = (A/B x C/D x 100) Where, A= Area of total impurity in test preparation B= Area of glycine in reference solution (a) C= Concentration of glycine in reference solution (a) D= Concentration of glycine in test solution Result: Impurity B = (_____________/_____________) x (____________/_____________) x 100 = _____________ Impurity H = (_____________/_____________) x (____________/_____________) x 100 = _____________ Impurity I = (_____________/_____________) x (____________/_____________) x 100 = ______________ Unspecified impurity = (____________/____________) x (___________/____________) x 100 = ___________ Total impurity = (____________/____________) x (___________/____________) x 100 = ___________ Acceptance criteria: impurity B: Max 0.10% impurity H: Max 0.10% impurity I: Max 0.10% Unspecified Impurity: Max 0.10% Total impurity: Max 0.2% |

| B. |  | As per USP: |
| --- | --- | --- |
| Instrument Used: Balance ID: __________________ Date of Calibration: ________Use Before: __________________ HPLC ID: __________________ Date of Calibration: ________Use Before: __________________ Solution / Reagents Used: ID of 5 N sodium hydroxide:_______________ Date of Preparation: __________Use Before: _____________ Solution A: Transfer 2.16 g of octane sulfonic acid sodium salt to a 1000 mL volumetric flask, add 900 mL of HPLC grade water and 2.0 mL of perchloric acid, and mix to dissolve. Adjust with 5 N sodium hydroxide solution to a pH of 2.2. Dilute with HPLC grade water to volume. Pass the solution through a membrane filter of 0.2-µm pore size. Solution B: Acetonitrile Mobile Phase: Gradient elution. See table 1 Time (Minutes); Solution A (%); Solution B (%). 0; 100; 0. 7; 100; 0. 13; 90; 10. 18; 90; 10. 35; 100; 0. 45; 100; 0. Standard solution: Solution ID:_____________________ Date of Preparation: ______________Use Before: _____________ Dilute 1.0 ml of above standard solution to 100 ml with water and mix well. (NOTE: Monochloroacetic acid may be omitted from the standard solution if the article being tested does not contain this substance.) Sample solution: Transfer _________(125.0) mg of glycine into a 25-mL volumetric flask, dissolve in and with HPLC grade water to volume. Blank: HPLC grade water Chromatographic system Mode: LC Detector: UV 200nm Column: 4.6-mm × 15-cm; 3-µm packing L1 Column temperature: 25° Flow rate: 1mL/min Injection volume: 20µL System suitability: Samples: Standard solution and Blank Suitability requirements: Interference peaks: Compare the chromatogram obtained from the standard solution with that obtained from the blank. Any peak area from the blank that overlaps or co-elutes with the amino acid peak from the standard solution is NMT 2.0 % of that amino acid peak area. Interference of Glycine in blank: __________ Interference of Diglycine in blank: __________ Interference of Triglycine in blank: __________ Interference of Glycine anhydride in blank: __________ Resolution: _________(NLT 2.0 between the diglycine and triglycine peaks, standard solution.) Relative standard deviation: _________(NMT 5.0 % each for the specified peaks, standard solution) Sequence: No.; Name of Solution; No. of Injections. 1.; Standard solution for system suitability; 1. 2.; Blank; 1. 3.; Standard solution; 5. 4.; Test solution; 2. Analysis: Standard solution, sample solution, and blank separately inject the blank, standard solution, and sample solution into the chromatograph. Compare the chromatogram from the sample solution with that from the blank. Disregard any peak observed in both the sample solution and the blank. Identify the amino acid impurities in the sample solution by comparing with those specified in the standard solution. Relative Retention Time: Glycine anhydride: 0.25, Monochloroacetic acid: 0.44, Iminodiacetic acid: 0.60, Glycine: 1.0, Diglycine: 1.70, Triglycine: 1.80, Hexamethylene tetramine: 2.47 Calculation: Result = (rU / rS) × (CS / CU) × 100 Where, ru = peak response of glycine anhydride, monochloroacetic acid, diglycine, or triglycine from the sample solution rs = peak response of glycine anhydride, monochloroacetic acid, diglycine, or triglycine from the standard solution Cs = concentration of glycine anhydride, monochloroacetic acid, USP Diglycine RS, or USP Triglycine RS in the Standard solution (mg/mL) CU = concentration of glycine in the sample solution (mg/mL) Separately calculate the percentage of iminodiacetic acid, hexamethylenetetramine, and any unspecified impurity in the portion of Glycine taken: Result = (rU/rS) × (CS /CU) × 100 rU = peak response of iminodiacetic acid, hexamethylenetetramine, or any unspecified impurity from the Sample solution rS = peak response of glycine from the Standard solution CS = concentration of USP Glycine RS in the Standard solution (mg/mL) CU = concentration of Glycine in the Sample solution (mg/mL) Result: Glycine anhydride = (____________/____________) x (_____________/___________) x 100 = ___________ Diglycine = (____________/____________) x (_____________/___________) x 100 = ___________ Triglycine = (____________/____________) x (_____________/___________) x 100 = ___________ Iminodiacetic acid = (____________/____________) x (_____________/___________) x 100 = ___________ Hexamethylenetetramine = (___________/___________) x (____________/__________) x 100 = __________ Monochloroacetic acid = (___________/___________) x (____________/__________) x 100 = __________ Unspecified impurity = (___________/___________) x (____________/__________) x 100 = __________ Total impurity = (___________/___________) x (____________/__________) x 100 = __________ Acceptance Criteria: Glycine anhydride: NMT 0.1% Monochloroacetic acid: Should be absent Iminodiacetic acid: NMT 0.1% Diglycine: NMT 0.1 % Triglycine: NMT 0.1% Hexamethylene tetramine: NMT 0.1% Unspecified impurity: NMT 0.1% Total impurities: NMT 1.0%. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________. |
| 12.0 |  | Ammonium (BP): |
| Test is carried out by outside laboratory using Amino acid analyzer. Report No.: ______________ Date of analysis: ________________ Observation: Ammonium: ___________ Acceptance criteria: NMT 0.02% Conclusion: Satisfactory / Not satisfactory Report reviewed by:____________ |  |  |
| 13.0 | Sulphate (USP): |  |
| Instrument Used: Balance ID: __________________ Date of Calibration: ________Use Before: __________________ Solution / Reagents Used: ID of 3N hydrochloric acid : _______________Date of Calibration: ____________Use Before: ____________ ID of barium chloride : _______________Date of Calibration: ____________Use Before: ____________ ID of 0.020 N sulfuric acid: _______________Date of Calibration: ____________Use Before: ____________ Procedure: Dissolve __________(3.0) g of Glycine 30 to 40 mL of water, if necessary, neutralize the solution with hydrochloric acid to litmus. Add 1 mL of 3 N hydrochloric acid, 3 mL of barium chloride TS, and sufficient water to make 50 mL Mix, and allow to stand for 10 minutes. Compare the turbidity, if any, with that produced in a solution containing the 0.20 mL of 0.020 N sulfuric acid. Observation: _________________________________________________________ Acceptance criteria: NMT 0.0065 % Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |
| 14.0 |  | Assay (IP/BP/USP): |
| A. |  | As per IP/USP |
| Instrument Used: Balance ID: __________________ Date of Calibration: ________Use Before: __________________ Volumetric Solution Used: ID of 0.1 N perchloric acid: ________________Date of Calibration: ___________Use Before: _____________ Procedure: Dissolve the 150.0 mg of Glycine, in 100 mL of glacial acetic acid, and add 1 drop of crystal violet TS, Titrate with the 0.1 N perchloric acid VS to a green endpoint. Perform a blank determination, and make any necessary correction. Each mL of 0.1 N per chloric acid is equivalent to 75.07 mg of C2H5NO2. Perform the titration in duplicate. Weight of set-1:_______________ Weight of set-2:_______________ Calculation: Assay = [(VS -VB) × N × F] × 100 × 100 W × (100 - LOD) x 0.1 VS = Titrant volume consumed by the sample (ml). VB = Titrant volume consumed by the blank (ml). N = Actual normality of the Titrant (mEq/ml) F = equivalency factor, 75.07 mg/mEq W = Sample weight (mg) Result: Assay-1 = [(________ -________) × ___________ × ___________] × 100 × 100 = ___________ __________ × (100 - __________) x 0.1 Assay-2 = [(________ -________) × ___________ × ___________] × 100 × 100 = ___________ __________ × (100 - __________) x 0.1 Mean: ___________ Acceptance criteria: 98.5% to 101.5% on the dried basis Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |

| B. | As per BP: |  |
| --- | --- | --- |
| Instrument Used: Balance ID: __________________ Date of Calibration: ________Use Before: __________________ Autotitrator ID: __________________ Date of Calibration: ________Use Before: __________________ Volumetric Solution Used: ID of 0.1 N perchloric acid: ________________Date of Calibration: ___________Use Before: _____________ Procedure: Dissolve 70.0 mg in 5 mL of anhydrous formic acid. Add 50 mL of anhydrous acetic acid. Immediately after dissolution, titrate with 0.1 M perchloric acid, determining the end-point potentiometrically. 1 mL of 0.1 M perchloric acid is equivalent to 7.51 mg of C2H5NO2. Weight of set-1:_______________ Weight of set-2:_______________ Result: Assay-1: _________________ Assay-2: ______________ Mean: ______________ Acceptance criteria: 98.5% to 101.0% (On the dried basis) Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |  |
| 15.0 |  | Sieve analysis: |
| Instrument Used: Balance ID: __________________ Date of Calibration: _______________ Use Before: _________________ Procedure: Assemble the sieves in consecutive order by opening size, with the coarsest sieve at the top, and place a solid-collecting pan below the bottom sieve. Place ________(100.0) g of the test sample, W, on the top sieve, and close the sieve with solid cover. Securely fasten the assembly to the sieve shaker, and operate the shaker for 15 minutes. Fix the sieve size as per customer requirement. After 15 minutes examine the sample. Calculate % passing using following formula. Calculation: Sieve; Weight Retained; Sample Weight; Passing Through (%); Result. 40; W1; S; {(S-W1) x 100} / S. 80; W2; S; {[S-(W1+W2)] x 100}/S. 100; W3; S; {[S-(W1+W2+W3)] x 100}/S. 150; W4; S; {[S-(W1+W2+W3+W4)] x 100}/S. 200; W5; S; {[S-(W1+W2+W3+W4+W5)] x 100}/S. Acceptance criteria: As per customer requirement. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |
| 16.0 |  | Bulk Density: |
| Instrument Used: Balance ID: __________________ Date of Calibration: _______________ Use Before: _________________ Procedure: Take 50 ml of measuring cylinder tare weight it and pass the powder to the measuring cylinder until 50 ml marked label. Weigh the measuring cylinder with powder fill up to 50 ml, and note the weight. Calculation: Bulk density = weight of sample (g)/ volume of sample (ml) Result: Weight of sample:________________ Bulk density = (_______________ / __________) = _______________ Acceptance criteria: As per customer requirement. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: ___________________ |  |  |

| 17.0 | Tapped density: |
| --- | --- |
| Instrument Used: Balance ID: __________________ Date of Calibration: _______________ Use Before: _________________ Procedure: Take 50 ml of measuring cylinder tare weight it and pass the powder to the measuring cylinder until 50 ml marked label. Tap the above measuring cylinder 100 times. Weigh the measuring cylinder after tapping 100 times, and note the weight. Calculation: Tapped density = weight of sample (g)/ volume of sample (ml) Result: Weight of sample:________________ Bulk density = (_______________ / __________) = _______________ Acceptance criteria: As per customer requirement. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |
| 18.0 | Elemental Impurities: |
| Instrument Used: Balance ID: __________________ Date of Calibration: _______________ Use Before: _________________ AAS ID: __________________ Date of Calibration: _______________ Use Before: _________________ Reagents / Solutions Used: Lead stock solution ID:_____________ Date of opening: _____________Use Before: _________________ Arsenic stock solution ID:_____________ Date of opening: _____________Use Before: _________________ Mercury stock solution ID:_____________ Date of opening: _____________Use Before: _________________ Cadmium stock solution ID:_____________ Date of opening: _____________Use Before: _________________ Cobalt stock solution ID:_____________ Date of opening: _____________Use Before: _________________ Vanadium stock solution ID:_____________ Date of opening: ____________Use Before: _________________ Nickle stock solution ID:_____________ Date of opening: ____________Use Before: _________________ Lithium stock solution ID:_____________ Date of opening: _____________Use Before: _________________ Antimony stock solution ID:_____________ Date of opening: ____________Use Before: _________________ Copper stock solution ID:_____________ Date of opening: _____________Use Before: _________________ Procedure: Sample Solution: Weigh _________(10) g of sample and transfer in 100 ml volumetric flask. Add 20 ml of Nitric acid and 20 ml of Hydrochloric acid. Digest the solution in digester. Make up the volume with water. Standard Solution: Stock Preparation: No.; Metal; Stock conc. in ppm; Volume of stock taken (ml); Final volume in water (ml); Final conc. in ppm. 1; Lead (Pb); 1000; 1; 100; 10. 2; Arsenic (As); 1000; 1; 100; 10. 3; Mercury (Hg); 1000; 1; 100; 10. 4; Cadmium (Cd); 1000; 1; 100; 10. 5; Cobalt (Co); 1000; 5; 100; 50. 6; Vanadium (V); 1000; 10; 100; 100. 7; Nickle (Ni); 1000; 20; 100; 200. 8; Lithium (Li); 1000; 25; 100; 250. 9; Antimony (Sb); 1000; 10; 100; 100. 10; Copper (Cu); 1000; 30; 100; 300. Calibration Curve Standard (Level-5): No.; Metal; Stock conc. in ppm; Volume of stock taken (ml); Final volume in water (ml); Final conc. in ppm. 1; Lead (Pb); 10; 5; 100; 0.5. 2; Arsenic (As); 10; 5; 100; 0.5. 3; Mercury (Hg); 10; 1; 100; 0.1. 4; Cadmium (Cd); 10; 5; 100; 0.5. 5; Cobalt (Co); 50; 10; 100; 5. 6; Vanadium (V); 100; 10; 100; 10. 7; Nickle (Ni); 200; 10; 100; 20. 8; Lithium (Li); 250; 10; 100; 25. 9; Antimony (Sb); 100; 9; 100; 9. 10; Copper (Cu); 300; 10; 100; 30. Calibration Curve Standard (Level-4): No.; Metal; Stock conc. in ppm; Volume of stock taken (ml); Final volume in water (ml); Final conc. in ppm. 1; Lead (Pb); 10; 2; 50; 0.4. 2; Arsenic (As); 10; 2; 50; 0.4. 3; Mercury (Hg); 10; 0.4; 50; 0.08. 4; Cadmium (Cd); 10; 2; 50; 0.4. 5; Cobalt (Co); 50; 4; 50; 4. 6; Vanadium (V); 100; 4; 50; 8. 7; Nickle (Ni); 200; 4; 50; 16. 8; Lithium (Li); 250; 4; 50; 20. 9; Antimony (Sb); 100; 3.5; 50; 7. 10; Copper (Cu); 300; 4; 50; 24. Calibration Curve Standard (Level-3): No.; Metal; Stock conc. in ppm; Volume of stock taken (ml); Final volume in water (ml); Final conc. in ppm. 1; Lead (Pb); 10; 1.5; 50; 0.3. 2; Arsenic (As); 10; 1.5; 50; 0.3. 3; Mercury (Hg); 10; 0.3; 50; 0.06. 4; Cadmium (Cd); 10; 1.5; 50; 0.3. 5; Cobalt (Co); 50; 3; 50; 3. 6; Vanadium (V); 100; 3; 50; 6. 7; Nickle (Ni); 200; 3; 50; 12. 8; Lithium (Li); 250; 3; 50; 15. 9; Antimony (Sb); 100; 2.5; 50; 5. 10; Copper (Cu); 300; 3; 50; 18. Calibration Curve Standard (Level-2): No.; Metal; Stock conc. in ppm; Volume of stock taken (ml); Final volume in water (ml); Final conc. in ppm. 1; Lead (Pb); 10; 1.0; 50; 0.2. 2; Arsenic (As); 10; 1.0; 50; 0.2. 3; Mercury (Hg); 10; 0.2; 50; 0.04. 4; Cadmium (Cd); 10; 1.0; 50; 0.2. 5; Cobalt (Co); 50; 2; 50; 2. 6; Vanadium (V); 100; 2; 50; 4. 7; Nickle (Ni); 200; 2; 50; 8. 8; Lithium (Li); 250; 2; 50; 10. 9; Antimony (Sb); 100; 1.0; 50; 2. 10; Copper (Cu); 300; 2; 50; 12. Calibration Curve Standard (Level-1): No.; Metal; Stock conc. in ppm; Volume of stock taken (ml); Final volume in water (ml); Final conc. in ppm. 1; Lead (Pb); 10; 0.5; 50; 0.1. 2; Arsenic (As); 10; 0.5; 50; 0.1. 3; Mercury (Hg); 10; 0.1; 50; 0.02. 4; Cadmium (Cd); 10; 0.5; 50; 0.1. 5; Cobalt (Co); 50; 1; 50; 1. 6; Vanadium (V); 100; 1; 50; 2. 7; Nickle (Ni); 200; 1; 50; 4. 8; Lithium (Li); 250; 1; 50; 5. 9; Antimony (Sb); 100; 0.5; 50; 1. 10; Copper (Cu); 300; 1; 50; 6.. Instrument Parameters: No.; Parameter; Value; No.; Parameter; Value. 1.; BGC Mode; BGC-D2; 4.; Slit width; As / Cd / Cu / Li / V / Hg: 0.7 nm Co / Sb / Ni: 0.2 nm Pb: 1.3 nm. 2.; Wavelength; Pb: 283.3 nm Li: 670.8 nm Cd: 228.8 nm Sb: 217.6 nm Co: 240.7 nm Ni: 232.0 nm Cu: 324.8 nm V: 318.4 nm As: 193.7 nm Hg: 253.7 nm; 5.; Flame type; As / Hg: HVG Mode Cd / Cu / Li / V / Co / Sb / Ni: Air – C2H2. 3.; Lamp current; As: 12 mA Cd: 8 mA Co: 12 mA Cu: 6 mA Li: 8 mA Pb: 12 mA Sb: 13 mA Ni: 12 mA V: 10 mA Hg: 12 mA; 6.; C2H2 flow rate; Cd/Cu/Li: 1.8 L/min Co/Ni: 1.6 L/min Pb/Sb: 2.0 L/min V: 7.0 L/min. 7.; Burner height; As/Hg: 16 mm V: 11 mm Cd/Co/Cu/Li/Pb/Sb/Ni: 7 mm. Procedure: Inject sample and standard solutions in above mentioned instrument condition for specific element. Record the absorbance for sample and standard solutions. Instrument shall calculate absorbance of element automatically from linearity of standard solutions. Calculate content of element using following formula. Calculation: Content (µg/g or ppm) = Content of specific element (µg/g or ppm) x SD / W Where, C: Content of element (µg/g or ppm) calculated by software SD: Volume W: Weight of sample Result: Lead (µg/g) = __________ x __________ / ___________ = ________________ Arsenic (µg/g) = __________ x __________ / ___________ = ________________ Mercury (µg/g) = __________ x __________ / ___________ = ________________ Cadmium (µg/g) = __________ x __________ / ___________ = ________________ Cobalt (µg/g) = __________ x __________ / ___________ = ________________ Lithium (µg/g) = __________ x __________ / ___________ = ________________ Antimony (µg/g) = __________ x __________ / ___________ = ________________ Vanadium (µg/g) = __________ x __________ / ___________ = ________________ Nickle (µg/g) = __________ x __________ / ___________ = ________________ Copper (µg/g) = __________ x __________ / ___________ = ________________ Acceptance criteria: Element; Class (ICH Q3D); Limit; Element; Class (ICH Q3D); Limit. Lead (Pb); I; NMT 0.5 µg/g; Vanadium (V); IIA; NMT 10.0 µg/g. Arsenic (As); I; NMT 0.5 µg/g; Nickle (Ni); IIA; NMT 20.0 µg/g. Mercury (Hg); I; NMT 0.1 µg/g; Lithium (Li); III; NMT 25.0 µg/g. Cadmium (Cd); I; NMT 0.5 µg/g; Antimony (Sb); III; NMT 9.0 µg/g. Cobalt (Co); IIA; NMT 5.0 µg/g; Copper (Cu); III; NMT 30.0 µg/g. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |
| 19.0 | Foreign matter: |
| Instrument Used: Balance ID: __________________ Date of Calibration: _______________ Use Before: _________________ Procedure: Take ________(5.0) g of Magnesium Aspartate sample and spread on a white paper uniformly. Observe material for absence of any foreign particle. Observation: __________________________________________________________________________. Acceptance criteria: Should be absent in 5 gm sample. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ |  |
| 20.0 | Residual Solvents: |
| Residual solvents is carried out by outside laboratory. Report No.:______________________ Date of analysis:_________________ Result: ___________ Acceptance criteria: Methanol: NMT 3000 ppm Conclusion: Satisfactory / Not satisfactory Report Reviewed By: ___________________ |  |
| 21.0 | Ethylene oxide: |
| This test is carried out by outside laboratory. Report No.: ______________________ Date of analysis: _________________ Result: _____________ Acceptance criteria: NMT 0.1 ppm Report reviewed by: _____________________ |  |
| 22.0 | Microbial contamination: |
| Instrument Used: Balance ID: __________________ Date of Calibration: _______________ Use Before: _________________ BOD Incubator ID: _____________ Date of Calibration: ______________ Use Before: ________________ Autoclave ID: _____________ Date of Calibration: ______________ Use Before: ________________ Colony counter ID: _____________ Date of Calibration: ______________ Use Before: ________________ A) Total aerobic microbial count: Procedure: Aseptically transfer powder equivalent to ___________(10.0) g to a sterile 100 ml volumetric flask, add about 70 ml of sterile phosphate buffer pH 7.2 and shake until an uniform suspension is obtained, dilute to 100 ml with phosphate buffer (I) (or Aseptically transfer 10gm to a sterile 90 ml casein soya bean digest broth pH 7.3 ± 0.20 and shake until an uniform suspension is obtained). Immediately Transfer aseptically 1 ml of (I) into each of two sterile Petri-dishes. Immediately add to each dish, 15 to 20 ml of Casein Soya bean digest agar medium that has been previously sterilized at 15 lbs pressure and cooled to 45°C. Cover the Petri-dishes, mix sample with the medium by tilting or rotating the Petri dishes and allow the contents to solidify at room temperature. Invert the Petri- dishes and incubate for 96 to 120 hours at 30°C to 35°C. After incubation examine the plates for growth. Count the number of colonies and express the average colonies of the two plates Average number of colonies multiplied by dilution factor will give number of cfu (microorganisms) present per gram. If no colonies are recovered from the plates representing 1:10 dilution of the sample, express the results as less than 10 cfu (microorganisms) per gram. Preparation of Reagents: - pH 7.2 Phosphate Buffer: Stock Solution ID: __________________ Date of Preparation: ____________ Use Before: _______________ Dilute the stock solution with purified water in the ration of 1:800, and sterilize. 2) Casein soya bean digest broth: Pancreatic digest of casein : 17.0 gm Papaic digest of Soya bean : 3.0 gm Sodium chloride : 5.0 gm Di Potassium hydrogen phosphate : 2.5 gm Glucose monohydrate : 2.5 gm Purified water : 1000 ml Adjust the pH so that after sterilization it is 7.3 ± 0.2 at 25°C Sterilize by heating in an autoclave at 121°C for 15 min. 3) Casein Soya bean Digest Agar Medium: Pancreatic Digest of Casein : 15.0 gm Papaic Digest of Soya bean meal : 5.0 gm Sodium Chloride : 5.0 gm Agar : 15.0 gm Water : 1000 ml pH after Sterilization : 7.3 ± 0.2 Observation: _________________________________ Acceptance criteria: NMT 103 CFU/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ B) Total combined yeasts/mould count: Procedure: Aseptically transfer powder equivalent to __________(10.0) g to a sterile 100 ml volumetric flask, add about 70 ml of sterile phosphate buffer pH 7.2 and shake until a uniform suspension is obtained, dilute to 100 ml with phosphate buffer (I). (or Aseptically transfer 10.0 g to a sterile 90 ml casein soya bean digest broth pH 7.3 ± 0.20 and shake until an uniform suspension is obtained,). (I). Immediately add to each dish, 15 to 20 ml of Sabouraud Dextrose Agar medium that has been previously sterilized at 15 lbs pressure and cooled to 45°C. Cover the Petri-dishes, mix sample with the medium by tilting or rotating the Petri-dishes, and allow the contents to solidify at room temperature. Invert the Petri-dishes and incubate for 5 days at 20°C to 25°C. After incubation examine the plates for growth. Count the number of colonies and express the average colonies of the two plates in number of fungi per gm. Average number of colonies multiplied by dilution factor will give number of fungi present per gm. If no colonies are recovered from the plates representing 1:10 dilution of the sample, express the results as less than 10 cfu (fungi) per gm. Sabouraud-dextrose agar Dextrose : 40.0 gm Mixture of peptic digest of animal : 10.0 gm Tissue and pancreatic digest of Casein (1:1) Agar : 15.0 gm Purified water : 1000ml Adjust the pH so that after sterilization it is 5.6 ± 0.2 at 25°C sterilize in anautoclave at 121°C for 15 min. Observation: ___________________________ Acceptance criteria: NMT 102 CFU/g Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ C) Pathogen in general: Escherichia coli: Aseptically transfer 10 ml of pre-treated sample 90 ml Casein soya bean digest broth and mix. Incubate at 30°C to 35°C for 18 to 24 hours. Examine the medium for growth, and if growth is present mix by gentle shaking. And transfer 1 ml of Casein soya bean digest broth to 100 ml of maccon-key broth, and incubate at 42°C to 44°C for 24 to 48 hours sub culture on a plate of macconkey agar at 30°C to 35°C for 18 to 72 hours. Growth of colonies indicates the possible presence of E.coli. This is confirmed by identification tests. The product complies with the test if no colonies are present or if the identification tests are negative. Mac Conkey broth: Pancreatic digest of gelatin : 20.0 gm Lactose monohydrate : 10.0 gm Dehydrate ox bile : 5.0 gm Bromocresol purple : 10.0 mg Purified water : 1000 ml Adjust the pH so that after sterilization it is 7.3 ± 0.2 at25°C. Sterilize in an autoclave at 121°C for 15 min. Mac Conky Agar Medium: Pancreatic Digest of Gelatin : 17.0 gm Pancreatic Digest of Casein : 1.5 gm Peptic Digest of Animal Tissue : 1.5 gm Lactose : 10.0 gm Bile Salts Mixture : 1.5 gm Sodium Chloride : 5.0 gm Agar : 13.5 gm Neutral Red : 30.0 mg Crystal Violet : 1.0 mg Purified water : 1000 ml Boil the mixture of solids and water for 1 minute to effect solution. pH after sterilization 7.1 ± 0.2. Observation: ___________________________ Acceptance criteria: Should be absent. Conclusion: Satisfactory / Not satisfactory Analyzed By: ___________________ Checked By: _____________________ Salmonella: Aseptically transfer 10 ml of pre-treated sample in 90 ml Casein soya bean digest broth and mix. Incubate at 30°C to 35°C. for18 to 24 hours. Transfer 0.1 ml of Casein soya bean digest broth to 10 ml of Rappaport Vas Siliadis Salmonella Enrichment Broth, and incubate at 30°C to 35°C for 18 to 24 hours sub culture on a plate of Xylose Lysine Deoxycholate Agar at 30°C to 35°C for 18 to 48 hours. Growth of colonies indicates the possible presence of salmonella. This is confirmed by identification tests (red colonies with or without black centres). The product complies with the test if no colonies are present or if the identification tests are negative. Dissolve warming slightly. Sterilize in an autoclave using a validated cycle, at a temperature not exceeding 115°. The pH is to be 5.2 ± 0.2 at 25° after heating and autoclaving. Adjust the pH so that after heating it is 7.4 ± 0.2 at 25°. Heat to boiling, cool to 50°, and pour into Petri dishes. Do not heat in an autoclave. Observation: _____________________ Acceptance criteria: Should be absent. Conclusion: Satisfactory / Not satisfactory Analyzed By:___________________ Checked By: _____________________ C) Bacterial endotoxin test: Procedure: Prepare the solutions and dilutions with water BET. If necessary, adjust the pH of the solution under examination to 6.0 to 8.0 using sterile 0.1 M hydrochloric acid BET, 0.1 M sodium hydroxide BET or a suitable buffer prepared with water BET. Preparations of test solutions: Prepare replicates of solutions A to D as indicated in the table. Solution; Final concentration of added CSE in the solution; Numbers of replicates. A; -; 4. B; 21; 4. B; 0.51; 4. B; 0.251; 4. C; 21; 4. C; 1; 2. C; 0.51; 2. C; 0.251; 2. D; -; 2. Solution A = Solution of the product at a dilution at or below MVD (test solution). Solution B = Test solution spiked with indicated CSE concentrations (Positive Product Control; PPC). Solution C = Standard solution with indicated CSE concentrations in water BET. Solution D = Water BET(Negative Control; NC). Method: Carry out the following procedure in receptacles such as tubes, vials or wells of micro-titre plates. Into each of the chosen receptacle, add an appropriate volume of negative control (NC), standard CSE solutions in water BET, test solution and positive product control (PPC). At intervals that will permit the reading of each result, add to each receptacle an equal volume of the appropriately constituted lysate unless single test vials are used. Mix the sample-lysate mixture gently and place in an incubating device such as a water-bath or a heating block, accurately recording the time at which the receptacles are so placed. Incubate each receptacle at 37º±1º undisturbed for 60±2 minutes. Remove the receptacles and examine the contents carefully. A positive reaction is characterised by the formation of a firm gel that retains its integrity when inverted through 180º in one smooth motion. Record this result as positive (+). A negative result is characterised by the absence of such a gel or by the formation of a viscous gel that does not maintain its integrity. Record such a result as negative (-). Handle the receptacles with care to avoid subjecting them to unwanted vibrations as false negative observations may result. Calculate the geometric mean end-point concentration of solutions of series B and C by using the formula described under Sensitivity of the lysate. Calculation and interpretation of results. The test for interfering factors is valid if a) solutions of series A and D give negative results; b) the results obtained with solutions of series C confirm the labelled sensitivity of the lysate; c) the geometric mean of the end-point concentration of solutions of series B is not more than 21 or not less than 0.51. If the result obtained is outside the specified limit, the test preparation under examination is acting as an inhibitor or activator. The interfering factors may be eliminated by further dilution (not greater than MVD), filtration, neutralisation, inactivation or by removal of the interfering substances. The use of a more sensitive lysate permits the use of greater dilution of the preparation under examination. Ultrafiltration may be used, if necessary, when the interfering factor passes through a filter with a nominal separation limit corresponding to a molecular weight of 10,000 to 20,000, such as asymmetrical membrane filters of cellulose triacetate. Such filters should be checked for the presence of any factors causing false positive results. The material retained on the filter, which contains the endotoxins, is rinsed with water BET or tris-chloride buffer pH 7.4 BET. The endotoxins are recovered in the water BET or the buffer. The endotoxin concentration in the test volume and the final volume are determined for each preparation under examination. Establish that the chosen treatment effectively eliminates interference without removing endotoxins by repeating the test for interfering factors using the preparation under examination to which the CSE has been added and which has been submitted to the chosen treatment. Observation: _____________________ Acceptance criteria: NMT 5 EU/g Conclusion: Satisfactory / Not satisfactory Analyzed By:___________________ Checked By: _____________________ |

Revision History:

| Protocol No. | Revision Made | Ref. CC No. | Effective Date |
| --- | --- | --- | --- |
| AWS/GCN/01(R-00) | New protocol prepared | --- |  |