| 1.0 |  | Description: |  |
| --- | --- | --- | --- |
| Procedure: Take about 1.0 g of sample and spread to clean Petri dish. Check product visually for colour. Acceptance Criteria: A white, crystalline powder. |  |  |  |
| 2.0 |  | Solubility: |  |
| Procedure: Take about 1.0 g of sample into 10 ml of water and check solubility. Take about 10 mg of sample into 100 ml of ethanol and check solubility. Take about 10 mg of sample into 100 ml of ether and check solubility. Acceptance criteria: Free soluble in water; very slightly soluble in ethanol (95%); practically insoluble in ether. |  |  |  |
| 3.0 |  | Identification: |  |
| Identification A: By IR Spectrophotometry Procedure: Blank preparation: Take 100 mg of KBr to be analysed. Load blank sample into DRS unit and take IR by following steps carried out for the background. Standard preparation: Prepare standard by mixing about 100 mg of KBr and 10 mg of standard. Load standard into DRS unit and take IR. Sample preparation: Prepare sample by mixing about 100 mg of KBr and 10 mg of substance to be analysed. Load sample into DRS unit and take IR. Procedure: Ensure that instrument is clean and free from dust. Switch ON the instrument and log in with respective ID as an authorized analyst. Set the desire instrument parameters. Allow the IR to stabilize IR takes some time for stabilization. After successful initialization, Install DRS unit in the IR. Clean the sample holder. Prepare fine powder of KBr in the mortar. Fill KBr powder into sample holder. Wipe excess powder with tissue paper and fix it to DRS unit. Select the method of background. Method parameters should be loaded and change the parameters as require. Now IR is ready take the KBr in sample holder and scan the background. Discard KBr and clean the unit, standard and sample Load into DRS unit and take IR by following steps. Select IR graph of standard and sample, Purity index or comparison spectra shall be opened as follow. Compare purity index with established limit to conclude the IR result. Acceptance criteria: IR spectrum of Glycine sample should be concordant with IR spectrum of Glycine working standard. Identification B: By Chemical test Procedure: Dissolve 50 mg in 5 ml of water, add 1 ml of sodium hypochlorite solution (3% Cl), boil for 2 minutes, add 1 ml of hydrochloric acid and boil for 4 to 5 minutes. To the resulting solution add 2 ml of hydrochloric acid and 1 ml of a 2% w/v solution of resorcinol, boil for 1 minute, cool, add 10 ml of water and mix. To 5 ml of this solution add 6 ml of 2 M sodium hydroxide. Acceptance Criteria: The resulting solution is violet with a greenish yellow fluorescence. After a few minutes the solution becomes orange and then yellow and the intense fluorescence remains. |  |  |  |
| 4.0 |  | Appearance of solution: |  |
| Procedure: Sample Solution: Dissolve 10 gm sample in 100 ml water. Standard Suspension: Dissolve 1.0 gm of hydrazine sulphate in sufficient water to produce 100 ml and set aside for about 6 hours. To 25 ml of this solution add 25 ml of a 10 % w/v solution of Hexamine, mix well and allow to stand for 24 hours. Keep in a glass container with a smooth internal surface In which the Suspension does not adhere to the glass. Store in this manner, the Suspension is stable for about 2 months. Prepare the diluted standard suspension by diluting 15 ml of the well mixed standard suspension to 1000 ml with water. The standard suspension should be used within 24 hours of preparation. Opalescence Standards: Prepare opalescence standards OS1 by mixing aliquots of standard suspension with water as indicated in Table 1. Each opalescence standard should be shaken well before use. Opalescence Standard Table 1: Opalescence Standard; Standard Suspension (ml); Water (ml). OS1; 5.0; 95.0. Procedure: Transfer to a flat-bottom test tube of neutral glass, 15 to 25 mm in diameter, a suitable volume of the solution under examination such that the test-tubes is filled to a depth of 4-tubes is filled to a depth of 40 mm. Into another matched test-tube add the same volume of the freshly prepared opalescence standard. After 5 minutes compare the contents of the test tubes against a black background by viewing under diffused light down the vertical axis of the tubes. Clarity of Opalescence: Express the degree of opalescence in terms of the opalescence standard. A liquid is considered as clear if its clarity is the same as that of water or of the solvent used for preparing the solution under examination or if its opalescence is not more than that of opalescence standard OS1. Colour of Solution Reagents: Ferric Chloride Colorimetric Solution (FCS) Dissolve 46 g of ferric chloride hexahydrate in enough of a mixture of 25 mL of hydrochloric acid and 975 mL of water to produce 1000 ml. Pipette 10 ml of this solution into a 250 ml iodine flask, add 15 ml of water, 3 g of potassium iodide and 5 ml of hydrochloric acid and allow the mixture to stand for 15 minutes. Dilute with 100 ml of water and titrate the liberated iodine with 0.1 M sodium thiosulphate using 0.5 ml of starch solution, added towards the end of the titration, as indicator. Carry out a blank titration. 1 mL of 0.1 M sodium thiosulfate is equivalent to 0.02703 g of FeCl3,6H2O. Adjust the final volume of the solution by the addition of enough of the mixture of hydrochloric acid and water so that each ml contains 0.045 g of FeCl3.6H2O. The solutions should be stored protected from light and standardised before use. Cobaltous Chloride Colorimetric Solution (CCS) Dissolve about 60 g of cobaltous chloride in enough of a mixture of 25 mL of hydrochloric acid and 975 mL of water to produce 1000 ml. Pipette 5 ml of this solution into a 250 ml iodine flask, add 5 ml of hydrogen peroxide solution (10 volume) and 15 ml of sodium hydroxide solution, boil for 10 minutes, cool and add 2 g of potassium iodide and 60 ml of dilute sulphuric acid. Dissolve the precipitate by gentle shaking, if necessary, and titrate the liberated iodine with 0.1 M sodium thiosulphate using 0.5 ml of starch solution, added towards the pink endpoint, as indicator. Carry out a blank titration. 1 mL of 0.1 M sodium thiosulfate is equivalent to 0.02379 g of CoCl2,6H2O. Adjust the final volume of the solution by the addition of enough of the mixture of hydrochloric acid and water so that each ml contains 0.0595 g of CoCl2,6H2O. Cupric Sulphate Colorimetric Solution (CSS) Dissolve about 63 g of cupric sulphate in enough of a mixture of 25 mL of hydrochloric acid and 975 mL of water to produce 1000 ml. Pipette 10 ml of this solution into a 250 ml iodine flask, add 40 ml of water, 4 ml of acetic acid, 3 g of potassium iodide and 5 ml of hydrochloric acid and titrate the liberated iodine with 0.1 M sodium thiosulphate using 0.5 ml of starch solution, added towards the pale brown end-point, as indicator. Carry out a blank titration 1 mL of 0.1 M sodium thiosulfate is equivalent to 0.02497 g of CuSO4,5H2O. Adjust the final volume of the solution by the addition of enough of the mixture of hydrochloric acid and water so that each ml contains 0.0624 g of CuSO4,5H2O. Reference Solution (YS7): Prepare by mixing FCS 0.8 ml, CCS 0.2 ml and hydrochloric acid (1%w/v) 99.0 ml. Method: Transfer to a flat bottom test tube of neutral glass 15 to 25 mm in diameter, a suitable volume of a liquid been examined such that the test tube is filled to a depth of 40 mm. Into another matched test tube add the same volume of water or the solvent used for preparing the solution being examined or of the reference solution stated in the individual monograph. Table 2. Colour of reference solution; Reference solution; FCS (ml); CCS (ml); CSS (ml); Hydrochloric Acid (1 per cent w/v HCL) (ml). Yellow; YS7; 0.8; 0.2; 0; 99.0. Acceptance criteria: A 10.0% w/v solution in carbon dioxide-free water is clear & not more intensely coloured than reference solution YS7. |
| 5.0 |  | pH: |  |
| Procedure: Determine the pH of 5% w/v solution of test sample on suitable pH meter. Acceptance criteria: 5.9 to 6.3 |  |  |  |
| 6.0 |  | Chlorides: |  |
| Procedure: Dissolve 2.5 g of the substance in 20 mL of water. Add 10 ml of dilute nitric acid, except when nitric acid is used in the preparation of the solution, dilute to 50 ml with water and add 1 ml of 0.1 M silver nitrate. Stir immediately with a glass rod and allow to stand for 5 minutes protected from light. When viewed transversely against a black background any opalescence produced is not more intense than that obtained by treating a mixture of 10.0 ml of chloride standard solution and 5 ml of water in the same manner. Acceptance Criteria: NMT 100 ppm |  |  |  |
| 7.0 |  | Heavy metals: |  |
| Procedure: Refer Method 14.0 Acceptance criteria: NMT 10 ppm |  |  |  |
| 8.0 |  | Sulphated Ash: |  |
| Procedure: Ignite a silica or platinum crucible at 600 ± 50º for 30 minutes, allow to cool in a desiccator over silica gel or other suitable desiccant and weigh. Place the prescribed amount of the substance under examination in the crucible and weigh. Moisten the substance under examination with a small amount of sulfuric acid (usually 1 ml) and heat gently at as low a temperature as practicable until the sample is thoroughly charred. After cooling, moisten the residue with a small amount of sulfuric acid (usually 1 ml), heat gently until white fumes are no longer evolved and ignite at 600 ± 50º until the residue is completely incinerated. Ensure that flames are not produced at any time during the procedure. Allow the crucible to cool in a desiccator over silica gel or other suitable desiccant, weigh it again and calculate the percentage of residue. If the amount of residue so obtained exceeds the prescribed limit, repeat the moistening with sulfuric acid and ignition, as previously, for 30 minutes’ periods until 2 consecutive weighing’s do not differ by more than 0.5 mg or until the percentage of residue complies with the prescribed limit. The amount of substance used for the test (usually 1-2 g) is chosen so that at the prescribed limit the mass of the residue (usually about 1 mg) can be measured with sufficient accuracy. Sulphated Ash = Weight of residue x 100 Weight of sample Acceptance Criteria: NMT 0.1 % |  |  |  |
| 9.0 |  |  | Loss on drying: |
| Procedure: Mix and accurately weigh the 1.0 g of the substance to be tested. Tare a glass-stoppered, shallow weighing bottle that has been dried for 30 minutes under the same conditions to be employed in the determination. Put the test specimen in the bottle, replace the cover, and accurately weigh the bottle and the contents. By gentle, sidewise shaking, distribute the test specimen as evenly as practicable to a depth of about 5 mm generally, and not more than 10 mm in the case of bulky materials. Place the loaded bottle in the drying chamber, removing the stopper and leaving it also in the chamber. Dry the test specimen at the 105o and for the 2 hours. Calculation: % LOD = Loss of weight x 100 / Weight of sample (g) Acceptance criteria: NMT 0.5 % |  |  |  |
| 10.0 | Assay: |  |  |
| Procedure: Dissolve 0.15 g in 100 ml of anhydrous glacial acetic acid. Immediately after dissolution titrate with 0.1 M perchloric acid, using 0.05 ml of crystal violet solution as indicator. Carry out blank titration. 1 mL of 0.1 M perchloric acid is equivalent to 0.00751 g of C2H5NO2. % Assay (on dried basis) = (VS – VB) × M × F × 100 x 100 0.1 x Sample weight in gm x (100 - % LOD) Acceptance criteria: 98.5 % to 101.5 % (on dried basis) |  |  |  |
| 11.0 | Sieve analysis: |  |  |
| Procedure: Assemble the sieves in consecutive order by opening size, with the coarsest sieve at the top, and place a solid- collecting pan below the bottom sieve. Place 100.0 g of the test sample, W, on the top sieve, and close the sieve with solid cover. Securely fasten the assembly to the sieve shaker, and operate the shaker for 15 minutes. After 15 minutes examine the sample. Acceptance criteria: As per customer requirement |  |  |  |
| 12.0 | Bulk Density: |  |  |
| Procedure: Take 50 ml of measuring cylinder tare weight it and pass the powder to the measuring cylinder until 50 ml marked label. Weigh the measuring cylinder with powder fill up to 50 ml, and note the weight. Calculation: weight of sample (g)/ volume of sample (ml) Acceptance criteria: As per customer requirement |  |  |  |
| 13.0 | Elemental Impurities: |  |  |
| Sample Solution: Weigh 10.0 g of sample and transfer in 100 ml dried volumetric flask. Add 20 ml of Nitric acid (1:1) and 20 ml of HCl (1:1). Digest the solution in digester. Make up the volume with water. Standard Solution: For Lead: Prepare 0.5 ppm, 0.4 ppm, 0.3 ppm, 0.2 ppm and 0.1 ppm solutions from lead 1000 ppm standard stock solution using water. For Arsenic: Prepare 0.5 ppm, 0.4 ppm, 0.3 ppm, 0.2 ppm and 0.1 ppm solutions from arsenic 1000 ppm standard stock solution using water. For Mercury: Prepare 0.1 ppm, 0.08 ppm, 0.06 ppm, 0.04 ppm and 0.02 ppm solutions from mercury 1000 ppm standard stock solution using water. For Cadmium: Prepare 0.5 ppm, 0.4 ppm, 0.3 ppm, 0.2 ppm and 0.1 ppm solutions from cadmium 1000 ppm standard stock solution using water. For Cobalt: Prepare 5 ppm, 4 ppm, 3 ppm, 2 ppm and 1 ppm solutions from cobalt 1000 ppm standard stock solution using water. For Vanadium: Prepare 10 ppm, 8 ppm, 6 ppm, 4 ppm and 2 ppm solutions from vanadium 1000 ppm standard stock solution using water. For Nickle: Prepare 20 ppm, 16 ppm, 12 ppm, 8 ppm and 4 ppm solutions from nickel 1000 ppm standard stock solution using water. For Lithium: Prepare 25 ppm, 20 ppm, 15 ppm, 10 ppm and 5 ppm solutions from lithium 1000 ppm standard stock solution using water. For Antimony: Prepare 9 ppm, 7 ppm, 5 ppm, 2 ppm and 1 ppm solutions from antimony 1000 ppm standard stock solution using water. For Copper: Prepare 30 ppm, 24 ppm, 18 ppm, 12 ppm and 6 ppm solutions from copper 1000 ppm standard stock solution using water. Instrument Parameters: No.; Parameter; Value; No.; Parameter; Value. 1.; BGC Mode; BGC-D2; 4.; Slit width; As / Cd / Cu / Li / V /Hg: 0.7nm Co / Sb / Ni:0.2 nmPb: 1.3 nm. 2.; Wavelength; Pb: 283.3 nm Li: 670.8 nm Cd: 228.8 nm Sb: 217.6 nm Co: 240.7 nm Ni: 232.0 nm Cu: 324.8 nm V: 318.4 nm As: 193.7 nm Hg: 253.7 nm; 5.; Flame type; As / Hg: HVG Mode Cd / Cu / Li / V / Co / Sb / Ni: Air – C2H2. 3.; Lamp current; As: 12 mA Cd: 8 mA Co: 12 mA Cu: 6 mA Li: 8 mA Pb: 12 mA Sb: 13 mA Ni: 12 mA V: 10 mA Hg: 12 mA; 6.; C2H2 flow rate; Cd/Cu/Li: 1.8 L/min Co/Ni: 1.6 L/min Pb/Sb: 2.0 L/min V: 7.0 L/min. 7.; Burner height; As/Hg: 16 mm V: 11 mm Cd/Co/Cu/Li/Pb/Sb/Ni: 7 mm. Procedure: Inject sample and standard solutions in above mentioned instrument condition for specific element. Record the content of element (µg/g or ppm). Instrument shall calculate content of element automatically from linearity curve of standard solutions. Calculate content of element in the sample taken using following formula. Calculation: Content (µg/g or ppm) = Content of specific element (µg/g or ppm) x SD / W Where, C: Content of element (µg/g or ppm) calculated by software SD: Volume W: Weight of sample Acceptance criteria: Lead (Pb); :; Lowest detection limit NMT 0.5 µg/g; Vanadium (V); :; Lowest detection limit NMT 0.5 µg/g. Arsenic (As); :; Lowest detection limit NMT 0.5 µg/g; Nickle (Ni); :; Lowest detection limit NMT 0.5 µg/g. Mercury (Hg); :; Lowest detection limit NMT 0.1 µg/g; Lithium (Li); :; Lowest detection limit NMT 0.5 µg/g. Cadmium (Cd); :; Lowest detection limit NMT 0.5 µg/g; Antimony (Sb); :; Lowest detection limit NMT 0.5 µg/g. Cobalt (Co); :; Lowest detection limit NMT 0.5 µg/g; Copper (Cu); :; Lowest detection limit NMT 0.5 µg/g. |
| 14.0 | Foreign matter: |  |  |
| Procedure: Take 5.0 g of sample and spread on a white paper uniformly. Observe material for absence of any foreign particle. Acceptance criteria: Should be absent in 5 gm sample. |  |  |  |
| 15.0 | OVI/Residual solvent: |  |  |
| Preparation of standard: Weigh 300 mg of methanol standard in volumetric flask of 100 ml, add 20 mL of Dimethyl Formamide and mix well and fill the volumetric flask up to the mark with Dimethyl Formamide and mix well. Preparation of Sample: Take 500 mg of test sample (Glycine) in volumetric flask of 100 ml, add 10 mL of Dimethyl Formamide and mix well and fill the volumetric flask up to the mark with Dimethyl Formamide and mix well. Procedure: Mode: GC Injector: Headspace Injection Type: Split ratio, 1:5 [Split ratio can be modified to optimize sensitivity]. Detector: Flame Ionization Column: 0.32-mm x 30-m capillary fused silica coated with 1.8 µm layer of phase G43 Column temperature: Initial Temperature (º); Temperature Ramp (% min); Final temperature (º); Hold time at Final Temperature (min). 40; 0; 40; 20. 40; 10; 240; ---. 240; 0; 240; 20. Temperatures Injection port:140º Detector: 250º Carrier gas: Helium, Nitrogen, or Hydrogen Linear velocity: About 35 cm/s for helium (adjustment and verification are necessary for other carrier gas) Calculation: % Methanol = Area of methanol in Glycine X Concentration of methanol in Standard X 106 _______________________ _______________________________ Area of methanol in Standard Concentration of methanol in sample Acceptance criteria: Methanol - NMT 3000 ppm |
| 16.0 | Ethylene oxide: |  |  |
| This test is carried out by outside laboratory using GC HS / GC MS. Acceptance criteria: NMT 0.1 ppm |  |  |  |
| 17.0 | Microbial enumeration tests: |  |  |
| A) Bacterial endotoxin test: Procedure: Prepare the solutions and dilutions with water BET. If necessary, adjust the pH of the solution under examination to 6.0 to 8.0 using sterile 0.1 M hydrochloric acid BET, 0.1 M sodium hydroxide BET or a suitable buffer prepared with water BET. Preparations of test solutions: Prepare replicates of solutions A to D as indicated in the table. Solution; Final concentration of added CSE in the solution; Numbers of replicates. A; -; 4. B; 21; 4. B; 0.51; 4. B; 0.251; 4. C; 21; 4. C; 1; 2. C; 0.51; 2. C; 0.251; 2. D; -; 2. Solution A = Solution of the product at a dilution at or below MVD (test solution). Solution B = Test solution spiked with indicated CSE concentrations (Positive Product Control; PPC). Solution C = Standard solution with indicated CSE concentrations in water BET. Solution D = Water BET(Negative Control; NC). Method: Carry out the following procedure in receptacles such as tubes, vials or wells of micro-titre plates. Into each of the chosen receptacle, add an appropriate volume of negative control (NC), standard CSE solutions in water BET, test solution and positive product control (PPC). At intervals that will permit the reading of each result, add to each receptacle an equal volume of the appropriately constituted lysate unless single test vials are used. Mix the sample-lysate mixture gently and place in an incubating device such as a water-bath or a heating block, accurately recording the time at which the receptacles are so placed. Incubate each receptacle at 37º ± 1º undisturbed for 60 ± 2 minutes. Remove the receptacles and examine the contents carefully. A positive reaction is characterised by the formation of a firm gel that retains its integrity when inverted through 180º in one smooth motion. Record this result as positive (+). A negative result is characterised by the absence of such a gel or by the formation of a viscous gel that does not maintain its integrity. Record such a result as negative (-). Handle the receptacles with care to avoid subjecting them to unwanted vibrations as false negative observations may result. Calculate the geometric mean end-point concentration of solutions of series B and C by using the formula described under Sensitivity of the lysate. Calculation and interpretation of results. The test for interfering factors is valid if a) solutions of series A and D give negative results; b) the results obtained with solutions of series C confirm the labelled sensitivity of the lysate; c) the geometric mean of the end-point concentration of solutions of series B is not more than 21 or not less than 0.51. If the result obtained is outside the specified limit, the test preparation under examination is acting as an inhibitor or activator. The interfering factors may be eliminated by further dilution (not greater than MVD), filtration, neutralisation, inactivation or by removal of the interfering substances. The use of a more sensitive lysate permits the use of greater dilution of the preparation under examination. Ultrafiltration may be used, if necessary, when the interfering factor passes through a filter with a nominal separation limit corresponding to a molecular weight of 10,000 to 20,000, such as asymmetrical membrane fIlters of cellulose triacetate. Such filters should be checked for the presence of any factors causing false positive results. The material retained on the filter, which contains the endotoxins, is rinsed with water BET or tris-chloride buffer pH 7.4 BET. The endotoxins are recovered in the water BET or the buffer. The endotoxin concentration in the test volume and the final volume are determined for each preparation under examination. Establish that the chosen treatment effectively eliminates interference without removing endotoxins by repeating the test for interfering factors using the preparation under examination to which the CSE has been added and which has been submitted to the chosen treatment. Acceptance Criteria: NMT 5 EU/gm Preparation of reagents: pH 7.2 Phosphate Buffer: Stock solution: Dissolve 34.0 g of monobasic potassium phosphate (KH2PO4) in about 500 ml of purified water contained in a 1000 ml volumetric flask. Adjust to pH 7.2 ± 0.20 by the addition of 1 N Sodium hydroxide (About 175 ml), add purified water to volume, mix and sterilize. Store under refrigeration. For use dilute the stock solution with purified water in the ration of 1:800, and sterilize. A) Total aerobic microbial count: Aseptically transfer powder equivalent to 10.0 g to a sterile 100 ml volumetric flask, add about 70 ml of sterile phosphate buffer pH 7.2 and shake until an uniform suspension is obtained, dilute to 100 ml with phosphate buffer (I) (or Aseptically transfer 10.0 g to a sterile 90 ml casein soya bean digest broth pH 7.3 ± 0.20 and shake until an uniform suspension is obtained). (I) Immediately Transfer aseptically 1 ml of (I) into each of two sterile Petri-dishes. Simultaneously put a negative control in which add 1 ml of sterile phosphate buffer. Immediately add to each dish, 15 to 20 ml of Casein Soya bean digest agar medium that has been previously sterilized at 15 lbs pressure and cooled to 45°C. Cover the Petri-dishes, mix sample with the medium by tilting or rotating the Petri dishes and allow the contents to solidify at room temperature. Invert the Petri-dishes and incubate for 96 to 120 hours at 30°C to 35°C. After incubation examine the plates for microbial growth. Count the number of colonies and express the average colonies of the two plates Average number of colonies multiplied by dilution factor will give number of cfu (microorganisms) present per gram. If no colonies are recovered from the plates representing 1:10 dilution of the sample, express the results as less than 10 cfu (microorganisms) per gram. Soya bean Casein digest broth Pancreatic digest of casein : 17.0 gm Papaic digest of Soya bean : 3.0 gm Sodium chloride : 5.0 gm Di Potassium hydrogen phosphate : 2.5 gm Glucose monohydrate : 2.5 gm Purified water : 1000 ml Adjust the pH so that after sterilization it is 7.3 ± 0.2 at 25°C. Sterilize in an autoclave using validate cycle. Soya bean Casein digest Agar Medium: Pancreatic Digest of Casein : 15.0 gm Papaic Digest of Soya bean meal : 5.0 gm Sodium Chloride : 5.0 gm Agar : 15.0 gm Water : 1000 ml Adjust the pH so that after sterilization it is 7.3 ± 0.2 at 25°C. Sterilize in an autoclave using validate cycle. Acceptance Criteria: NMT 103 CFU/g. B) Total combined yeasts/mould count: Procedure: Aseptically transfer powder equivalent to 10.0 g to a sterile 100 ml volumetric flask, add about 70 ml of sterile phosphate buffer pH 7.2 and shake until an uniform suspension is obtained, dilute to 100 ml with phosphate buffer (I). (or aseptically transfer 10gm to a sterile 90 ml casein soya bean digest broth pH 7.3 ± 0.20 and shake until an uniform suspension is obtained). (I) Immediately Transfer aseptically 1 ml of (I) into each of two sterile Petri-dishes. Simultaneously put a negative control in which add 1 ml of sterile phosphate buffer. (I) Immediately add to each dish, 15 to 20 ml of Sabouraud Dextrose Agar medium that has been previously sterilized at 15 lbs pressure and cooled to 45°C. Cover the Petri-dishes, mix sample with the medium by tilting or rotating the Petri-dishes, and allow the contents to solidify at room temperature. Invert the Petri-dishes and incubate for 5 days at 20°C to 25°C. After incubation examine the plates for growth. Count the number of colonies and express the average colonies of the two plates in number of fungi per gm. Average number of colonies multiplied by dilution factor will give number of fungi present per gm. If no colonies are recovered from the plates representing 1:10 dilution of the sample, express the results as less than 10 cfu (fungi) per gm. Sabouraud-dextrose agar: Dextrose : 40.0 gm Mixture of peptic digest of animal : 10.0 gm Tissue and pancreatic digest of Casein (1:1) Agar : 15.0 gm Purified water : 1000 ml Adjust the pH so that after sterilization it is 5.6 ± 0.2 at 25°C Sterilize in an autoclave using validate cycle. Acceptance Criteria: NMT 102 CFU/g. C) Pathogen in general: Escherichia coli: Aseptically transfer 10 ml of pre-treated sample in 90 ml Soya bean Casein digest broth and mix. Incubate at 30°C to 35°C. for 18 to 24 hours. Examine the medium for growth, and if growth is present mix by gentle shaking. And transfer 1 ml of Soya bean Casein digest broth to 100 ml of MacConkey broth, and incubate at 42°C to 44°C for 24 to 48 hours sub culture on a plate of MacConkey agar at 30°C to 35°C for 18 to 72 hours. Growth of colonies indicates the possible presence of E. coli (brick red). This is confirmed by identification tests. The product complies with the test if no colonies are present or if the identification tests are negative. MacConkey broth: Pancreatic digest of gelatin : 20.0 gm Lactose monohydrate : 10.0 gm Dehydrate ox bile : 5.0 gm Bromocresol purple : 10.0 mg Purified water : 1000 ml Adjust the pH so that after sterilization it is 7.3 ± 0.2 at 25°C. Sterilize in an autoclave using a validated cycle MacConkey Agar Medium: Pancreatic Digest of Gelatin : 17.0 gm Pancreatic Digest of Casein : 1.5 gm Peptic Digest of Animal Tissue : 1.5 gm Lactose : 10.0 gm Bile Salts Mixture : 1.5 gm Sodium Chloride : 5.0 gm Agar : 13.5 gm Neutral Red : 30.0 mg Crystal Violet : 1.0 mg Purified water : 1000 ml Boil the mixture of solids and water for 1 minute to effect solution. Sterilize in an autoclave using a validated cycle pH after sterilization 7.1 ± 0.2. Acceptance criteria: Should be absent. Salmonella: Aseptically transfer 10 ml of pre-treated sample in 90 ml Soya bean Casein digest broth and mix. Incubate at 30°C to 35°C. for18 to 24 hours. Transfer 0.1 ml of Soya bean Casein digest broth to 10 ml of Rappaport Vassiliadis Salmonella Enrichment Broth, and incubate at 30°C to 35°C for 18 to 24 hours sub culture on a plate of Xylose Lysine Deoxycholate Agar at 30°C to 35°C for 18 to 48 hours. Growth of colonies indicates the possible presence of salmonella. This is confirmed by identification tests (red colonies with or without black centres). The product complies with the test if no colonies are present or if the identification tests are negative. Rappaport Vassiliadis Salmonella Enrichment Broth: Soya Peptone :4.5 g Magnesium Chloride Hexahydrate :29.0 g Sodium Chloride :8.0 g Dipotassium Phosphate :0.4 g Potassium Dihydrogen Phosphate :0.6 g Malachite Green :0.036 g Purified Water :1000 mL Dissolve warming slightly. Sterilize in an autoclave using a validated cycle, at a temperature not exceeding 115°. The pH is to be 5.2 ± 0.2 at 25° after heating and autoclaving. Xylose Lysine Deoxycholate Agar: Xylose :3.5 g L-Lysine :5.0 g Lactose Monohydrate :7.5 g Sucrose :7.5 g Sodium Chloride :5.0 g Yeast Extract :3.0 g Phenol Red :80 mg Agar :13.5 g Sodium Deoxycholate :2.5 g Sodium Thiosulfate :6.8 g Ferric Ammonium citrate :0.8 g Purified Water :1000mL Adjust the pH so that after heating it is 7.4 ± 0.2 at 25°. Heat to boiling, cool to 50°, and pour into Petri dishes. Do not heat in an autoclave. Acceptance criteria: Should be absent. Pseudomonas aeruginosa: Aseptically transfer 10 ml of pre-treated sample in 90 ml Soya bean Casein digest broth and mix. Incubate at 30°C to 35°C. for 18 to 24 hours. Examine the medium for growth, if growth is present, mix by gentle shaking. And subculture on the plate of Cetrimide agar at 30°C to 35°C for 18 to 24 hours. Growth of colonies indicates the possible presence of Pseudomonas. This is confirmed by identification tests. The product complies with the test if no colonies are present or if the identification tests are negative. Cetrimide Agar Medium: Pancreatic Digest of Gelatin : 20.0 gm Magnesium Chloride : 1.4 gm Dipotassium Sulfate : 10.0 gm Cetrimide : 0.3 gm Agar : 13.6 gm Purified water : 1000 ml Glycerol : 10.0 mL Heat to boiling for 1 minute with shaking. Adjust the pH so that after sterilization it is 7.2±0.2 at 25o C. Sterilize by Autoclave. Acceptance criteria: Should be absent. Staphylococcus aureus: Aseptically transfer 10 ml of pre-treated sample in 90 ml Soya bean Casein digest broth and mix. Incubate at 30°C to 35°C. for 18 to 24 hours. Examine the medium for growth, if growth is present, mix by gentle shaking. And subculture on the plate of Mannitol agar at 30°C to 35°C for 18 to 24 hours. Growth of colonies indicates the possible presence of Staphylococcus. This is confirmed by identification tests. The product complies with the test if no colonies are present or if the identification tests are negative. Mannitol agar Medium: Pancreatic Digest of Casein : 5.0 gm Paptic Digest of Animal tissue : 5.0 gm Beef Extract : 1.0 gm D- Mannitol : 10.0 gm Sodium Chloride : 75.0 gm Agar : 15.0 gm Phenol red :0.025 gm Purified water : 1000 ml Heat to boiling for 1 minute with shaking. Adjust the pH so that after sterilization it is 7.4 ± 0.2 at 25o C. Sterilize by Autoclave. Acceptance criteria: Should be absent. Coliforms: Aseptically transfer 10 ml of pre-treated sample in 90 ml Soya bean Casein digest broth and mix. Incubate at 30°C to 35°C. for 18 to 24 hours. Examine the medium for growth, if growth is present, mix by gentle shaking. And subculture on the LST broth tubes containing Durham’s vial at 35°C for 24 hours. Gas production indicates the possible presence of Coliforms. This is confirmed by identification tests. The product complies with the test if no colonies are present or if the identification tests are negative. Lauryl Tryptose Broth: Tryptose : 20 g Lactose : 5 g Dipotassium phosphate : 2.75 g Monopotassium phosphate : 2.75 g Sodium chloride : 5 g Sodium lauryl sulfate : 0.1 g Water : 1000 mL Adjust the pH so that after sterilization it is 6.8 ± 0.1 at 25o C. Sterilize by Autoclave. Acceptance criteria: Should be absent. |

Revision History:

| Specification No. | Revision Made | Ref. CC No. | Effective Date |
| --- | --- | --- | --- |
| MOA/FG00038/01 (R-00) | New MOA prepared | ------- | 21/04/2021 |
| MOA/FG00038/01 (R-01) | Format revised as per revised SOP QC 12. Some contents revised as per FSAAI requirement. | CC/2022/03 | 01/06/2022 |
| MOA/FG00038/01(R-02) | Elemental impurity by AAS added. Ethylene oxide test added. | CC/2023/010 | 01/06/2023 |